Date:  05/14/06

Products Affected:  VENDING MACHINE, LOCKER, VACUVEND, TOOLROOM

Summary:  HOST DOWN TROUBLESHOOTING SCRIPT FOR LANTRONIX DEVICE

Description of Issue:  KEYPAD DISPLAYS “HOST DOWN” MESSAGE

The following conditions should trigger this script:

- Keypad displays “Host Down” and the customer is using Lantronix devices to poll machines

Solution:  Perform the script to isolate any possible software configuration problems or hardware malfunctions.  Each step is a requirement for the next step.  If you can’t successfully complete a step, stop until that problem is remedied.

Time Required:  30 Minutes

PERSONNEL PERFORMING THIS SCRIPT MUST BE AUTOCRIB CERTIFIED

I.  Keypad displays “Host Down”

- Is Autopoll running and responsive?

  - NO:  Restart Autocrib

    - If machines begin to poll, attempt test issue.  Otherwise follow next step below.

  - YES:  Skip to next step.

- Does the controller name match the computer name?

  - NO:  Insert the new controller name in the AutoCrib software and reassign the terminals.  (The old controller must be deleted prior to reassigning the terminals to the new controller name.)

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Is the port set to ‘1’ in the Controllers tab of the AutoCrib software?

  - NO:  Edit port number in Tools\Options\Controllers of AutoCrib software.

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Are there other devices connected to the same port?

  - NO:  Skip to next step

  - YES:  Change the port number through Device Manager for the other device.

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

- Can you ping and telnet the MSS?

  - NO:

    - Are the MSS and computer in the same IP scheme?

      - NO:   Contact IT to place in same IP scheme.

        - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

      - YES:  Follow next step

    - Does the network admin allow telnets and pings on network?

      - NO:  Remove management station and MSS from network and try to ping and telnet MSS.

        - If telnet and ping are successful, bring UDS up to location of MSS and attempt to create session between the two devices.

          - If session has been created, place UDS back in original place.  If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

          - If session was not created, reset and reprogram MSS device.

            - If unable to reprogram MSS,

              - Verify serial port A is enabled in BIOS

              - Verify comport is OK

              - New MSS may be needed if all vending machines are down.

            - If able to reprogram MSS but machines are not polling, skip to next step.

            - If machines being to poll, attempt test issue.  Otherwise, skip to next step.

      - YES:  Reset and reprogram MSS device.

        - If machines begin to poll, attempt test issue.

        - If unable to reprogram MSS,

          - Verify serial port A is enabled in BIOS

          - Verify comport is OK

          - New MSS may be needed

        - If able to reprogram MSS but machines are not polling, skip to next step.

  - YES:  Skip to next step.

- Is the UDS in the hostlist of the MSS?

  - NO:  Add UDS to hostlist.  Then, perform “ID0” command.

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Does the MSS have a session with the UDS?

  - NO:  Perform “set priv” and “ID0” command.  Wait for a few minutes for connection to reestablish.

    - Does the MSS have a session with the UDS?

      - NO:  Bring UDS up to location of management station and MSS and attempt to create session with the device.

        - If session has been created, place UDS back in original place.

          - If machines begin to poll, attempt test issue.

          - If machines do not begin to poll,

            - Verify pinout of DB25 connector

            - UDS IP address, gateway, netmask, and settings

            - Have IT verify the network port is OK.  Otherwise, skip to next step.

        - If session was not created,

          - Reprogram UDS

          - UDS may need to be replaced.

      - YES:  If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Follow next step.

NOTE:  THE MSS MUST HAVE A SESSION WITH THE UDS BEFORE CONTINUING FURTHER!

- Are the terminal addresses correct on each keypad?

  - NO:  Correct each terminal address that is incorrect.  If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Is the RDU connected to a battery backup?

  - NO:  Skip to next step.

  - YES:  Remove the machine(s) from the battery backup and plug directly into wall outlet.  If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

- Remove all RDUs from daisy chain and/or home run.  Is the RDU polling?

  - YES:  Begin to connect one RDU at a time until the system goes down.  Then, locate the short.  The troubleshooting steps below can be used to find short.

  - NO:  Skip to next step.

- Is the line straight through inside the vending machine and through each daisy chain link?

  - NO:  Replace the bad cable.  If machines begin to poll; attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Bypass the splitter by connecting the CAT5 cable directly from the DB25 connector to the controller.  Is the RDU polling?

  - NO:  Skip to next step.

  - YES:  Purchase new splitter

- Check motor count.  Is the motor count correct?

  - NO:  Find short in machine.

    - Vending Machines:  Remove all shelves from backbone cable and disconnect any additional pieces of hardware such as lockers to a vending machine.  Connect one shelf at a time to backbone and perform motor count.  Once the motor count is incorrect, the drawer must be removed from the machine and the wires must be inspected.

    - AutoLockers:  View locker troubleshooting guide.

  - YES:  Replace keypad/controller set.