Date:  05/14/06

Products Affected:  VENDING MACHINE, LOCKER, VACUVEND, TOOLROOM

Summary:  HOST DOWN TROUBLESHOOTING SCRIPT FOR SEALEVEL DEVICE

Description of Issue:  KEYPAD DISPLAYS “HOST DOWN” MESSAGE

The following conditions should trigger this script:

- Keypad displays “Host Down” and the customer is using Sealevel devices to poll machines

Solution:  Perform the script to isolate any possible software configuration problems or hardware malfunctions.  Each step is a requirement for the next step.  If you can’t successfully complete a step, stop until that problem is remedied.

Time Required:  30 Minutes

PERSONNEL PERFORMING THIS SCRIPT MUST BE AUTOCRIB CERTIFIED

I.  Keypad displays “Host Down”

- Is Autopoll running and responsive?

  - NO:  Restart Autocrib

    - If machines begin to poll, attempt test issue.  Otherwise follow next step below.

  - YES:  Skip to next step.

- Does the controller name match the computer name?

  - NO:  Insert the new controller name in the AutoCrib software and reassign the terminals.  (The old controller must be deleted prior to reassigning the terminals to the new controller name.)

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Is there a solid EN light on the Sealevel device?

  - NO:

    - Are the Sealevel drivers installed on the computer?

      - NO:   Install the drivers.

        - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

      - YES:  Try using a different USB port on the pc.

        - If EN light will not appear, replace the Sealevel device.

  - YES:  Skip to next step.

- Does the port in the Controllers tab match the port in the Device Manager

  - NO:  Edit port number in Tools\Options\Controllers of AutoCrib software to match the port number in the Device Manager.

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Are there other devices connected to the same port?

  - NO:  Skip to next step

  - YES:  Change the port number through Device Manager for the other device.

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

- Are the TX and RX lights blinking?

  - NO:  Verify the following.

    - DB25 connector has correct pinout

    - CAT5 cable is connected to Sealevel device, busblock, and RDU.

    - If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Recycle power on RDU.

    - If machines begin to poll, attempt test issue.  Otherwise skip to next step.

- Are the terminal addresses correct on each keypad?

  - NO:  Correct each terminal address that is incorrect.  If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Is the RDU connected to a battery backup?

  - NO:  Skip to next step.

  - YES:  Remove the machine(s) from the battery backup and plug directly into wall outlet.  If machines begin to poll, attempt test issue.  Otherwise, skip to next step.

- Remove all RDUs from daisy chain and/or home run.  Is the RDU polling?

  - NO:  Skip to next step.

  - YES:  Begin to connect one RDU at a time until the system goes down.  Then, locate the short.  The troubleshooting steps below can be used to find short.

- Is the line straight through inside the vending machine and through each daisy chain link?

  - NO:  Replace the bad cable.  If machines begin to poll; attempt test issue.  Otherwise, skip to next step.

  - YES:  Skip to next step.

- Bypass the splitter by connecting the CAT5 cable directly from the DB25 connector to the controller.  Is the RDU polling?

  - NO:  Skip to next step.

  - YES:  Purchase new splitter

- Check motor count.  Is the motor count correct?

  - NO:  Find short in machine.

    - Vending Machines:  Remove all shelves from backbone cable and disconnect any additional pieces of hardware such as lockers to a vending machine.  Connect one shelf at a time to backbone and perform motor count.  Once the motor count is incorrect, the drawer must be removed from the machine and the wires must be inspected.

    - AutoLockers:  View locker troubleshooting guide.

  - YES:  If all vending machines will not poll, replace Sealevel device.  Otherwise, replace keypad/controller set.